Annex 1a - Technical specification for part 1 of public contract

Technical specification

Test equipment for ADAS systems, including control PC

The subject of performance for part 1 of the public contract is the delivery of test equipment for ADAS systems, including control PC. The intent is the acquisition of HIL test equipment for validation of ADAS systems in automotive. The equipment will focus primarily on the validation of image sensors (camera). The test equipment must contain test and simulation HW and SW. One part must be SW for modelling the surrounding scene of a vehicle and simulation of driving scenarios. Transport to the place of performance and operator training is also part of the subject of performance.

Part 1) Testing tools for ADAS systems including user PC (1 piece / set):

  - Modular HW supporting testing and simulations;

  - SW package contains tools for configuration of HW (in a)), modelling and simulation of surrounding scenarios;

  - User PC;

  - Basic training course with the delivered SW tools.

Manufacturer of the device:			to be completed by contracting party

Exact type designation of the device:	to be completed by contracting party

Testing tools for ADAS systems including user PC must meet following criteria:

| Basic technical parameters | Minimal required values – obligatory condition! | Value of the offered device |
| --- | --- | --- |
| - Modular testing and simulation HW |  |  |
| Modular solution | Yes | the participiant indicates YES/NO |
| Real-Time CPU unit | Yes, min. 2,5 GHz | the participiant indicates YES/NO and indicates specific value |
| Cores number of CPU unit | min. 4 | the participiant indicates YES/NO and indicates specific value |
| RAM memory | min. 4 GB DDR4 | the participiant indicates YES/NO and indicates specific value |
| PCI Express interface | Min. 2 | the participiant indicates YES/NO and indicates specific value |
| Ethernet interface | Yes, 1 Gbit/s | the participiant indicates YES/NO |
| Active cooling of CPU | Yes | the participiant indicates YES/NO |
| Cover for modular HW | 19“ rack | the participiant indicates YES/NO |
| Heigh of cover for modular HW | min. 12 HU | the participiant indicates YES/NO and indicates specific value |
| Modul(es) general I/O with automotive range – Analog inputs | Yes, min. 10 inputs with range 0 .. 40 V, 14 bit AD converter | the participiant indicates YES/NO |
| Modul(es) general I/O with automotive range – Digital inputs | Yes, min. 10 inputs with range 0 .. 40 V | the participiant indicates YES/NO |
| Modul(es) general I/O with automotive range – Analog outputs | Yes, min. 10 inputs with range 0 .. 10 V, 14 bit DA converter | the participiant indicates YES/NO |
| Modul(es) general I/O with automotive range – Digital outputs | Yes, min. 10 inputs with range 5 .. 40 V | the participiant indicates YES/NO |
| Resistance simulation | Yes, min. 4 channels | the participiant indicates YES/NO and indicates specific value |
| Modul(es) for automotive communication buses | Yes, CAN/CAN FD, Automotive ethernet | the participiant indicates YES/NO |
| Number of channels for CAN/CAN FD bus | Min. 4 | the participiant indicates YES/NO and indicates specific value |
| Support of CAN-bus protocols | ISO 11898, J1939 | the participiant indicates YES/NO |
| Terminating resistors | Yes, for each channel | the participiant indicates YES/NO |
| Wake-up on CAN and Sleep modes support | Yes | the participiant indicates YES/NO |
| Number of possible channels for automotive ethernet | Min. 4 | the participiant indicates YES/NO and indicates specific value |
| Ethernet baud rate | 10/100/1000 Mbit/s | the participiant indicates YES/NO |
| Number of installed channels for automotive ethernet | Min. 2 | the participiant indicates YES/NO and indicates specific value |
| BroadR-Reach, 100Base-T1, 1000Base-T1 compatible | Yes | the participiant indicates YES/NO |
| Modul for simulation (PC) of ADAS applications | Yes | the participiant indicates YES/NO |
| Modul for simulation (PC) – 19“ rack mount | Yes | the participiant indicates YES/NO |
| Modul for simulation (PC) – CPU | Yes, min. 3,5 Ghz | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – CPU power | PassMark Average CPU Mark minimal value 9000 according to http://www.cpubenchmark.net on the day of processing, resp. Submission of the offers | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – RAM memory | min. 16 GB DDR4 | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – HDD | min. 256 GB, SSD | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – Operating system | Windows 10 | the participiant indicates YES/NO |
| Modul for simulation (PC) – Video output | min. 1x Display Port, 1x HDMI | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – Graphic adapter | extern, non-sharing memory min. 24 GB, GDDR6 | the participiant indicates YES/NO and indicates specific value |
| Modul for simulation (PC) – Interfaces | Min. 1x Ethernet 1 Gbit/s | the participiant indicates YES/NO |
| - SW package contains tools for configuration of HW, modelling and simulation of surrounding scenarios |  |  |
| SW tools for configuration of HW in a) | Yes | the participiant indicates YES/NO |
| Configuration SW tools – configuration of I/O channels | Yes | the participiant indicates YES/NO |
| Configuration SW tools – MATLAB/Simulink compatibility | Yes | the participiant indicates YES/NO |
| Configuration SW tools – AUTOSAR support | Yes | the participiant indicates YES/NO |
| Configuration SW tools – CAN-Bus, Automotive ethernet configuration | Yes | the participiant indicates YES/NO |
| Control and simulation SW tools for HW in a) | Yes | the participiant indicates YES/NO |
| Control and simulation SW tools – support for CAN/CAN-FD (analysis, simulation) | Yes | the participiant indicates YES/NO |
| Control and simulation SW tools – support for Automotive ethernet (analysis, simulation) | Yes | the participiant indicates YES/NO |
| Control and simulation SW tools – support for HW electrical signals (analysis, simulation) | Yes | the participiant indicates YES/NO |
| Pre-created simulation models – compatible with MATLAB/Simulink | Yes | the participiant indicates YES/NO |
| Simulation models compatible with HW in a) for HiL simulation and testing | Yes | the participiant indicates YES/NO |
| General simulation model for vehicle dynamic simulation | Yes | the participiant indicates YES/NO |
| General simulation model of the vehicle environment enabling modelling of vehicle environment scenes for the development of ADAS systems (other vehicles, traffic objects, pedestrians, etc.) | Yes | the participiant indicates YES/NO |
| Modelling SW tool for editing simulation models and their parameterization | Yes | the participiant indicates YES/NO |
| Graphical interface of the modelling tool | Yes | the participiant indicates YES/NO |
| Simulation tool for implementation of simulation models | Yes | the participiant indicates YES/NO |
| Simulation tool for simulation of ADAS sensors | Yes | the participiant indicates YES/NO |
| Creating custom scenes in a simulation tool | Yes | the participiant indicates YES/NO |
| Rendering a 3-D image of the surrounding scene | Yes | the participiant indicates YES/NO |
| The colour range of the image | RGB, 24bit | the participiant indicates YES/NO |
| Generating an image in the form of a fisheye | Yes | the participiant indicates YES/NO |
| Movement of objects in the scene with respect to the laws of physics | Yes | the participiant indicates YES/NO |
| Simulation of various surfaces of objects (reflectivity, etc.) | Yes | the participiant indicates YES/NO |
| Tool for creating automated test tasks compatible with HW in point a) | Yes | the participiant indicates YES/NO |
| - User PC |  |  |
| CPU | x86-64 compatible | the participiant indicates YES/NO |
| CPU Power | PassMark Average CPU Mark minimal value 28000 according to http://www.cpubenchmark.net on the day of processing, resp. Submission of the offers | the participiant indicates YES/NO and indicates specific value |
| RAM memory | min. 32 GB DDR4, 2900 MHz | the participiant indicates YES/NO and indicates specific value |
| Hard disk | min. 1 TB SSD | the participiant indicates YES/NO and indicates specific value |
| Graphic adapter | extern, non-sharing memory min. 8 GB, GDDR6 | the participiant indicates YES/NO and indicates specific value |
| Power supply | min. 750 W | the participiant indicates YES/NO and indicates specific value |
| Optical drive | Yes | the participiant indicates YES/NO |
| Monitor 1 – diagonal | min. 27“ | the participiant indicates YES/NO and indicates specific value |
| Monitor 1 – resolution | min. 2560 x 1440, IPS | the participiant indicates YES/NO and indicates specific value |
| Monitor 1 – vertically adjustable base | Yes | the participiant indicates YES/NO |
| Monitor 2 – diagonal | min. 27“ | the participiant indicates YES/NO and indicates specific value |
| Monitor 2 - resolution | min. 2560 x 1440, IPS | the participiant indicates YES/NO and indicates specific value |
| Monitor 2 - vertically adjustable base | Yes | the participiant indicates YES/NO |
| Mouse, optical, cable connection | Yes | the participiant indicates YES/NO |
| Keyboard, cable connection | Yes | the participiant indicates YES/NO |
| - Basic training course SW tools |  |  |
| Number of persons | Min. 2 | the participiant indicates YES/NO |
| Number of training days | Min. 1 | the participiant indicates YES/NO |
| Online/attendance training | Yes | the participiant indicates YES/NO |
| Language of the course | English or Czech | the participiant indicates YES/NO |

The participant should enter the data in compliance with the technical specification of the offered devices.