Annex 1b - Technical specification for part 2 of public contract

Technical specification

Interface for scene injection on camera image detector

The subject of performance for part 2 of the public contract is the delivery of an interface for scene injection on camera image sensor. The intent is the acquisition of HIL test equipment for validation of ADAS systems in automotive. The equipment should focus primarily on the validation of image sensors (camera). The test equipment must contain test and simulation HW and SW. One part must be SW for modelling the surrounding scene of a vehicle and simulation of driving scenarios. Transport to the place of performance is also part of the subject of performance.

Part 2) Interface for scene injection on camera image detector (1 piece / set):

  - Tool for emulation of image sensor.

Manufacturer of the device:			to be completed by contracting party

Exact type designation of the device:	to be completed by contracting party

T Interface for scene injection on camera image detector must meet following criteria:

| Basic technical parameters | Minimal required values – obligatory condition! | Value of the offered device |
| --- | --- | --- |
| - Tool for emulation of image sensor |  |  |
| Input video interface from simulation PC | Min. Display Port | the participiant indicates YES/NO |
| Image resolution | Min. Full HD 1920x1080 | the participiant indicates YES/NO and indicates specific value |
| Possibility to connect multiple cameras in parallel | Yes, min. 2 | the participiant indicates YES/NO and indicates specific value |
| Real-time image processing | Yes | the participiant indicates YES/NO |
| Communication protocol for image data injection | Parallel DVP/Serial MIPI CSI2 | the participiant indicates YES/NO |
| HDR support | Yes | the participiant indicates YES/NO |
| Modular solution | Yes | the participiant indicates YES/NO |
| Compatibility with standard dSpace / Vector / NI / MicroNova test devices | Yes | the participiant indicates YES/NO |
| Compatibility with cameras of German OEM vehicle manufacturers | Yes, MFK3/MFK4 | the participiant indicates YES/NO |

The participant should enter the data in compliance with the technical specification of the offered devices.