Technická specifikace

Nákup „3D tiskárny“ typu delta, technologie LDM určená pro tisk porcelánu, keramiky, žáromateriálů s velikostí trysek od 1-8 mm, kde součástí tiskárny je kontinuální plnící jednotka, ruční plnící jednotka materiálu vše kompatibilní s tiskárnou.

Purchase of a "3D printer" delta type, LDM technology designed for printing of ceramic, porcelain, refractories, with nozzle size from 1 to 8 mm, where the printer includes a continuous feeding system compatible with the printer.

Základní (minimální) technické parametry

Účastník uvede splnění veškerých základních technických parametrů stanovených zadavatelem v tomto dokumentu formou popisu technického řešení v nabídce, resp. uvede hodnoty jednotlivých technických parametrů nabízeného zařízení do tabulky Základní technické specifikace nabízeného zařízení. Technické specifikace je nutno řešit v plném rozsahu, a to pouze jedním dedikovaným zařízením, nabídky účastníka budou vyhodnocovány jako oddělené nabídky v případě, že by se rozhodl nabídnout různá řešení

Základní technická specifikace nabízeného zařízení

Účastník uvede do tabulky níže hodnoty jednotlivých technických parametrů nabízeného zařízení a předloží jí jako součást své nabídky.

Minimální technické parametry VZ:

| Parameters | Contracting authority requirement | Nabízený parametr |
| --- | --- | --- |
| 3D print system | Delta |  |
| Materials for print | ceramic, porcelain, earthenware, gres, refractory |  |
| Technology | Liquid Deposition Modeling |  |
| Print volume | ⌀ 400 mm x 1000 mm |  |
| Nozzle diameter | 1-8 mm |  |
| Travel speed | max 150 mm/s |  |
| Print speed | max 150 mm/s |  |
| Layer resolution | Min. 0.5 mm to 5 mm |  |
| Srew extruder | extruder features a stainless-steel extrusion screw that enables printing with retractions |  |
| Huge prints | 1m |  |
| Remote control | can be connected to the network, mobile phone, tablet or computer |  |
| Input | Input: 220/240 V, 50/60 Hz |  |
| Power consumption | 200 W |  |
| Operating Systems | Windows, Mac, Linux |  |
| Simplify 3D File exstension: | .stl, .obj, .gcode |  |
| Slicing softwares: | Cura, SLic3r, |  |
| Temperature | 0-30°C |  |
| Operating language | English, Czech |  |
| Operating Systems | Windows, Mac, Linux |  |
| Simplify 3D File exstension | .stl, .obj, .gcode |  |
| Slicing softwares: | Cura, SLic3r, |  |
| Temperature | 0-30°C |  |
| Continuous Feeding System (CFS) | compatible with printer |  |
| Materials for Continous Feeding System (CFS) | porcelain, earthenware, gres, refractory |  |
| Frame and cover | stainless steel |  |
| Compatible nozzles diameter | 4 – 6 – 8 mm |  |
| Continuous feeding control unit | yes |  |
| Layer resolution | from 0.5 mm to 5 mm |  |
| Capacity | max. 12 liters |  |
| Power requirements | Input: 220/240 V 50/60 Hz |  |
| Absorbed power | Max. 500 W |  |
| Max output | max. 30 kg/h |  |
| Enviroment requirements | 0-30 C° |  |
| Operating language | English, Czech |  |
| Tools & Replacements | - 1x clay kit - 1x extra extruder (for printing as ceramics, nozzle 1-8) - 1x kit of nozzles for manual feeding system - 1x nozzle kit for 3D print ceramic (diameter 4,6,8 mm), - 1x manual feeding extruder for LDM - 2x screw for LDM (for ceramic) - 1x 5l alumina tank (safety valve set at 7 bar) for ceramic - 1x 3 l alumina tank (safety valve set at 7 bar) for ceramic - 2X POM screw for manual feeding system - 2X screw for LDM (for ceramic) - 2x POM screw for LDM (for ceramic) - 2x clay mixtures |  |

V ……………………………

Jméno a funkce osoby oprávněné jednat za dodavatele……………………………………………………..

Podpis………………………………………………………