Technical Specification

120° high resolution long-range LiDAR sensor

The subject of the public contract is the delivery of a set of 4 pcs of 120° high resolution long-range LiDAR (Light Detection and Ranging) sensors. The performance also includes transport to the place of performance. All sensors in the set shall be identical.

120° high resolution long-range LiDAR sensor manufacturer: 	to be completed by the tenderer

Exact type designation of the 120° high resolution long-range LiDAR sensor:	to be completed by the tenderer

Number of pieces: 	4

The 120° high resolution long-range LiDAR sensor must have at least the following components and must fulfil at least the following parameters:

| Basic technical parameters | Minimum required values - must be met! | Value of the offered device |
| --- | --- | --- |
| Range (@10% reflectivity) | Min. 180 m | the tenderer shall indicate YES/NO and the specific value of the offered device |
| Field of view (horizontal × vertical) | 120°x25° | the tenderer shall indicate YES/NO |
| Weight (sensor only) | <1 000 g | the tenderer shall indicate YES/NO and the specific value of the offered device |
| Power consumption (nominal) | <19 W | the tenderer shall indicate YES/NO |
| Points per second (Single return) | Min. 1 500 000 pts/s | the tenderer shall indicate YES/NO and the specific value of the offered device |
| Frame rate (configurable) | 10 Hz/20 Hz | the tenderer shall indicate YES/NO |
| Operating voltage | Min. 9-16 V | the tenderer shall indicate YES/NO |
| Application area | Automotive industry | the tenderer shall indicate YES/NO |
| Scanning method | Rotating mirror | the tenderer shall indicate YES/NO |
| Number of channels | Min. 128 | the tenderer shall indicate YES/NO and the specific value of the offered device |
| Operating temperature | Min. –40 ℃ to +85 ℃ | the tenderer shall indicate YES/NO |
| Time synchronization | PTP (IEEE 1588 nebo 802.1AS) | the tenderer shall indicate YES/NO |
| Laser safety class | Class 1 – Eye-safe | the tenderer shall indicate YES/NO |
| Data per point | Range, azimuth angle and reflectivity | the tenderer shall indicate YES/NO |
| Range accuracy | Max. ±5 cm | the tenderer shall indicate YES/NO |
| Ingress protection | Min. IP6K7 | the tenderer shall indicate YES/NO |

The data shall be completed by the tenderer in accordance with the technical data of the offered device.