Annex No. 1 - Technical Specification

Technical Specification

Expansion of the platform for testing and advanced control methods (HIL/RCP)

The subject matter of the public contract is the delivery of hardware modules intended for the expansion of the existing HIL testing equipment for the validation of electronic systems.

The resulting setup will enable the stimulation and analysis of electrical I/O signals, provide communication interfaces, and emulate fault states of control units (ECUs). It will support rest-bus simulation and automatically execute real-time tests. The equipment replaces real sensors, actuators, and the communication of surrounding control systems so that it is possible to verify the functionality and behavior of the examined systems on new hardware topologies and algorithms without the need to use a real controlled system.

The software part, together with the HW components, must form a comprehensive package for the modeling, simulation, configuration, and testing of electronic control systems, including the creation of rest-bus simulations, supplementary models, and automated tests. It must include a tool for simulating and testing control systems and networks with support for CAN, CAN FD, Ethernet, LIN, and other communication interfaces, enabling automated testing, analysis of measured values, and full configuration and control of the supplied HW part. It must also include a tool for creating automated tests in both textual (C#, CAPL) and graphical formats; the created tests must be exportable and compatible with the simulation tool, and variables and signals must be transferable in a unified format. The setup is complemented by software for administration, configuration, calibration, and other supporting functions necessary for the complete operation of the SW and HW as a whole.

The subject of performance also includes delivery to the place of performance.

The module overview specification below defines the parameters for assembling the validation set intended for testing various applications. The requirements for the set are as follows:

| Requirement | Quantity | Required values | Value of the offered equipment |
| --- | --- | --- | --- |
| Subrack housing - Number of slots in the subrack housing for modules | 2 | 19'', number of modules 12 | the participant shall state YES/NO |
| Rack power supply module | 1 | YES – max 1 slot | the participant shall state YES/NO |
| RT module | 2 | YES – max 2 slots | the participant shall state YES/NO |
| Communication module for CAN | 6 | to provide 24 channels | the participant shall state YES/NO |
| Communication module for LIN | 1 | to provide 4 channels | the participant shall state YES/NO |
| Ethernet communication module | 3 | to provide 14 channels | the participant shall state YES/NO |
| SUT power supply module | 1 | YES – max 1 slot | the participant shall state YES/NO |
| USB communication module CAN/LIN | 4 | to provide 16 channels | the participant shall state YES/NO |
| CAN communication piggy | 34 | to provide 34 channels | the participant shall state YES/NO |
| LIN communication piggy | 4 | to provide 4 channels | the participant shall state YES/NO |
| Ethernet communication piggy | 3 | to provide 14 channels | the participant shall state YES/NO |
| CYESe Licence | 1 | Upgrade of existing license to v19 | the participant shall state YES/NO |
| CYESe option J1939 Licence | 1 | v19 | the participant shall state YES/NO |
| CYESe option LIN Licence | 1 | v19 | the participant shall state YES/NO |
| CYESe option C2X Licence | 1 | Upgrade of existing license to v19 | the participant shall state YES/NO |
| CYESe option Ethernet Licence | 1 | v19 | the participant shall state YES/NO |
| vTESTstudio Licence | 1 | v10.0 | the participant shall state YES/NO |

The existing HIL testing equipment for the validation of electronic systems owned by the contracting authority is based on HW and SW from Vector Informatik. The contracting authority requires that the offered/supplied expansion sets be compatible with the VTsystem product line and the CYESe, vTESTstudio SW tools, and their supplementary tools. Within the performance of the public contract, existing components will be utilized, and the functionalities of the setup will be enhanced by the required expansion sets. As part of the SW delivery, an update of current licenses and the addition of new licenses for selected tools are required - see the table above starting with the CYESe license (the last 6 rows of the table).

Equipment manufacturer: 			to be stated by the participant

Exact type designation of the equipment:	to be stated by the participant

The equipment must meet the following criteria:

| Basic technical parameters | Minimum required values – must be met! | Value of the offered equipment |
| --- | --- | --- |
| - HW modules |  |  |
| - Module subrack housing |  |  |
| Supported number of modules: 12 | YES | the participant shall state YES/NO |
| Possibility of expansion with an additional subrack housing to increase the number of modules | YES | the participant shall state YES/NO |
| Dimension for rack cabinet integration: 19'' | YES | the participant shall state YES/NO |
| - Housing power supply module |  |  |
| Mains supply: 230 VAC | YES | the participant shall state YES/NO |
| Manual switch to turn on the housing power supply | YES | the participant shall state YES/NO |
| Maximum dimensions | 1 module slot | the participant shall state YES/NO |
| - Power supply module for the system under test (SUT) |  |  |
| Input voltage | -40 ÷ +40 V | the participant shall state YES/NO and specify the exact value |
| Current capacity per channel | min. 60 A | the participant shall state YES/NO and specify the exact value |
| Possibility of using an integrated power supply powered from the housing | YES | the participant shall state YES/NO |
| Measurement of output current and voltage | YES | the participant shall state YES/NO |
| Measurement of output current and voltage | YES | the participant shall state YES/NO |
| Number of output channels | min. 2 | the participant shall state YES/NO and specify the exact value |
| Possibility of short-circuiting output channels | YES | the participant shall state YES/NO |
| Possibility of connecting an external power supply | YES | the participant shall state YES/NO |
| Possibility of controlling an external power supply | YES | the participant shall state YES/NO |
| - Real-time simulation module (RT module) |  |  |
| CPU | 2.7 GHz, 9MB cache, 6 cores, 12 threads, 14nm tech. | the participant shall state YES/NO and specify the exact value |
| RAM | min. 32 GB | the participant shall state YES/NO and specify the exact value |
| USB | min. 4x USB 3.0 | the participant shall state YES/NO and specify the exact value |
| PCI express | min. 8x PCIe Gen 3 x1 | the participant shall state YES/NO and specify the exact value |
| LAN | YES – ethernet | the participant shall state YES/NO |
| Support for simulation with a 1ms time cycle | YES | the participant shall state YES/NO |
| Support for Extended Realtime with a 500µs cycle time | YES | the participant shall state YES/NO |
| - Communication module for CAN and LIN buses |  |  |
| Number of channels | min. 4 | the participant shall state YES/NO and specify the exact value |
| Max. CAN baud rate | 1 Mbit/s | the participant shall state YES/NO and specify the exact value |
| Connection interface | PCIe Gen 3 | the participant shall state YES/NO and specify the exact value |
| Timestamp accuracy | max. 1 µs | the participant shall state YES/NO and specify the exact value |
| Possibility of fault state emulation, i.e., short circuit of signal wires to Vcc, GND, etc. | YES | the participant shall state YES/NO |
| Minimum number of CAN, CAN FD channels | 16 | the participant shall state YES/NO and specify the exact value |
| Minimum number of LIN channels | 2 | the participant shall state YES/NO and specify the exact value |
| - Communication module for Ethernet bus |  |  |
| Number of channels | min. 6 | the participant shall state YES/NO and specify the exact value |
| Supported protocol | IEEE 100BASE-T1 | the participant shall state YES/NO and specify the exact value |
| Connection interface | PCIe Gen 3 | the participant shall state YES/NO and specify the exact value |
| Timestamp accuracy | max. 1 µs | the participant shall state YES/NO and specify the exact value |
| - Additional parameters and modules |  |  |
| Compatibility with VTsystem series modules | YES | the participant shall state YES/NO |
| Possibility of connecting additional communication modules to the RT module via USB interface | YES | the participant shall state YES/NO |
| Portable CAN/LIN communication module with 4 communication channels | YES | the participant shall state YES/NO |
| - SW resources |  |  |
| - SW package for simulation and testing of electronic systems |  |  |
| License type: Standard (without restrictions) | YES | the participant shall state YES/NO |
| License version: v19 | YES | the participant shall state YES/NO |
| Number of licenses: 1 | YES | the participant shall state YES/NO |
| Simulation of communication buses | YES | the participant shall state YES/NO |
| Implementation of databases for buses: DBC, LDF, ARXML | YES | the participant shall state YES/NO |
| Textual display of measured values | YES | the participant shall state YES/NO |
| Graphical display of measured values | YES | the participant shall state YES/NO |
| Implementation of automated tests | YES | the participant shall state YES/NO |
| UDS diagnostic functions | YES | the participant shall state YES/NO |
| Stimulation of communication buses | YES | the participant shall state YES/NO |
| HIL implementation | YES | the participant shall state YES/NO |
| CYESe option CAN a CAN FD - update of existing license from v17 to v19 | YES | the participant shall state YES/NO |
| CYESe option J1939 | YES | the participant shall state YES/NO |
| CYESe option LIN | YES | the participant shall state YES/NO |
| CYESe option Ethernet | YES | the participant shall state YES/NO |
| CYESe option C2X - update of existing license from v17 to v19 | YES | the participant shall state YES/NO |
| 2.2. SW package for creating automated tests |  |  |
| Direct compatibility with item 1.1. | YES | the participant shall state YES/NO |
| Direct compatibility with item 2.1. | YES | the participant shall state YES/NO |
| Textual creation of test tasks | YES | the participant shall state YES/NO |
| Programming language: C#, CAPL | YES | the participant shall state YES/NO |
| Graphical creation of test tasks | YES | the participant shall state YES/NO |
| License type: Standard (without restrictions) | YES | the participant shall state YES/NO |
| Number of licenses: 1 | YES | the participant shall state YES/NO |

The data shall be filled in by the supplier in accordance with the technical specifications of the offered equipment.