Annex No. 1 – Technical specifications

Technical specifications

HW/SW development platform for HIL applications

- The subject of this Public Contract is the supply of a development platform for HIL applications. The platform shall consist of:

- HW elements in the form of programmable control units for applications for automotive electronic systems - supplied with a connector set,

- SW elements in the form of an extension of the development environment for the creation of so-called model-based algorithms – user licenses must be included (at least 8 user licenses) and

- Calibration environment – user licenses must be included (at least 8 user licenses).

The subject of performance also includes transport to the place of performance.

Manufacturer of the device:				to be completed by the participant

Exact type designation of the device:		to be completed by the participant

HW/SW development platform for HIL applications must meet following criterias:

| Basic technical parameters | Minimal required values – obligatory condition! | Value of the offered device |
| --- | --- | --- |
| 1 pc - Programmable control Unit intended primarily for the Control of internal combustion Engines |  |  |
| Max. Current for Actuator Supplies | Min. 2x 10 A | the participiant indicates YES/NO and indicates specific value |
| Reference Voltage for Sensor Supplies | Min. 4 x 5 V | the participiant indicates YES/NO and indicates specific value |
| Communication Interface | Min. 4 x CAN 2.0 | the participiant indicates YES/NO and indicates specific value |
| Communication Interface SENT | YES | the participiant indicates YES/NO |
| Digital Inputs | Min. 8 | the participiant indicates YES/NO and indicates specific value |
| Analog Inputs | Min. 17 | the participiant indicates YES/NO and indicates specific value |
| Inputs for Temperature Sensors | Min. 9 | the participiant indicates YES/NO and indicates specific value |
| Knock Sensors Inputs | Min. 2 | the participiant indicates YES/NO and indicates specific value |
| Inputs for Lambda Sensors | Min. 4 | the participiant indicates YES/NO and indicates specific value |
| Supply Voltage 6,5 – 36 V | YES | the participiant indicates YES/NO |
| H-Bridges | Min. 2x 10 A | the participiant indicates YES/NO and indicates specific value |
| Current Strain of Outputs | Min. 4 x 0.2 A, 5 x 0. 5 A, 2 x 6 A/4 A, 1 x 8 A | the participiant indicates YES/NO and indicates specific value |
| Controlled Current Outputs | Min. 8 x 2 A | the participiant indicates YES/NO and indicates specific value |
| Outputs for Ignition Coil Control | Min. 8x 5 V | the participiant indicates YES/NO and indicates specific value |
| Power Supply parameters for controlling injector solenoids (minimum Requirements) | - 8 x 25 A / 15 A, - Configurable Voltage up to 65 V, - Power min. 120W. | the participiant indicates YES/NO and indicates specific value |
| 1 pc - Programmable control Unit intended primarily for so-called supervisory Control of electric and hybrid Vehicle Vehicles |  |  |
| Unit equipped with primary and secondary Processor | YES | the participiant indicates YES/NO |
| Supply Voltage 6.5–18 V | YES | the participiant indicates YES/NO |
| Reference Voltage for Sensor Supplies | Min. 2x 5 V | the participiant indicates YES/NO and indicates specific value |
| Inputs | Min. 40 | the participiant indicates YES/NO and indicates specific value |
| Outputs | Min. 50 | the participiant indicates YES/NO and indicates specific value |
| Communication Interface | Min. 4 x CAN for primary Processor Min. 1x CAN for secondary Processor | the participiant indicates YES/NO and indicates specific value |
| H-Bridges | Min. 1x 10 A, 2x 5 A, 1x 3.2 A | the participiant indicates YES/NO and indicates specific value |
| Current Strain of Outputs | Min. 11x 100 mA, 4x 400 mA, 14x 700 mA, 2x 1 A, 4x 2.2 A, 1x 3. 2 A | the participiant indicates YES/NO and indicates specific value |
| Protection against Moisture and Dust: IP69K | YES | the participiant indicates YES/NO |
| 2 pcs – Programmable control Unit |  |  |
| Supply Voltage 7 V - 32 V | YES | the participiant indicates YES/NO |
| Reference Voltage for Sensor Supplies | Min. 1 x 5 V / 250 mA | the participiant indicates YES/NO and indicates specific value |
| Communication Interface | Min. 2x CAN 2.0 | the participiant indicates YES/NO and indicates specific value |
| Analog Inputs | Min. 12 | the participiant indicates YES/NO and indicates specific value |
| Frequency Inputs | Min. 1 | the participiant indicates YES/NO and indicates specific value |
| Digital Inputs | Min. 4 | the participiant indicates YES/NO and indicates specific value |
| H-Bridges | Min. 1 x 5 A | the participiant indicates YES/NO and indicates specific value |
| Current Strain of Outputs | Min. 2 x 100 mA, 6 x 2 A, 2 x 250 mA | the participiant indicates YES/NO and indicates specific value |
| Output for Injectors activation | Min. 1 x 15 A / 5 A | the participiant indicates YES/NO and indicates specific value |
| 1 pc – Programmable control Unit |  |  |
| Reference Voltage for Sensor Supplies | Min. 1 x 5 V | the participiant indicates YES/NO and indicates specific value |
| Communication Interface CAN | Min. 2x | the participiant indicates YES/NO and indicates specific value |
| SENT Rx | Min. 1x | the participiant indicates YES/NO and indicates specific value |
| Analog Inputs | Min. 3x | the participiant indicates YES/NO and indicates specific value |
| Current Strain of Outputs | Min. 2x 100 mA, 6x 500 mA, 4x 2 A | the participiant indicates YES/NO and indicates specific value |
| Analog Output 2 x 0–5 V, 10 mA | YES | the participiant indicates YES/NO |
| Supply Voltage 6–36 V | YES | the participiant indicates YES/NO |
| Temperature Range for Unit operation | ISO 16750-4 (-40 °C to 85 °C) | the participiant indicates YES/NO and indicates specific value |

The participiant should enter the data in compliance with the technical specification of the offered devices.