Annex No. 1 – Technical specifications

Technical specifications

Measuring platform for analysis of radio interface and signaling in 5G technology

- The subject of the public contract is the supply of hardware and software part of measuring platform for analysis of radio interface and signaling in 5G technology.

The subject of performance also includes transportation to the place of performance.

Manufacturer of the device:				to be completed by the participant

Exact type designation of the device:		to be completed by the participant

Measuring platform for analysis of radio interface and signaling in 5G technology must meet following criterias:

| Basic technical parameters | Minimal required values – obligatory condition! | Value of the offered device |
| --- | --- | --- |
| 1) Hardware part: |  |  |
| Handheld hardware terminal supporting 5G NR radio interface technology | YES | the participiant indicates YES/NO |
| Support at least 3GPP radio band n78 | YES | the participiant indicates YES/NO |
| The terminal chipset supports communication with the software specified in point 2) | YES | the participiant indicates YES/NO |
| The terminal can be connected, controlled and supported by the software installed on the notebook/ PC mentioned in point 2) | YES | the participiant indicates YES/NO |
| 2) Software part: |  |  |
| Air interface test tool for 5G NR real-time analysis | YES | the participiant indicates YES/NO |
| Installed for notebook/PC with Windows operating system | YES | the participiant indicates YES/NO |
| Supporting 5G NR technology | YES | the participiant indicates YES/NO |
| Allowing to perform measurements with connected 5G Hardware specified in 1) and other terminals based on supported chipsets | YES | the participiant indicates YES/NO |
| Support for detailed L3 signaling analysis | YES | the participiant indicates YES/NO |
| Allowing to record measurements as log files | YES | the participiant indicates YES/NO |
| Allowing to play previously recorded logfiles | YES | the participiant indicates YES/NO |
| The flexible licensing system (license can be easily used on another notebook/PC) | YES | the participiant indicates YES/NO |

The participiant should enter the data in compliance with the technical specification of the offered devices.