Annex No. 1 – Technical specifications

Technical specifications

I/O modules for simulator running in realtime

- The subject of the public contract is the supply of I/O modules for simulator running in realtime.

The subject of performance also includes transportation to the place of performance, installation (HW + SW), commissioning and burning of modules.

Manufacturer of the device:				to be completed by the participant

Exact type designation of the device:		to be completed by the participant

I/O modules for simulator running in realtime must meet following criterias:

| Basic technical parameters | Minimal required values – obligatory condition! | Value of the offered device |
| --- | --- | --- |
| HW compatibility with HIL simulator from Speedgoat | YES | the participiant indicates YES/NO |
| 2 pcs of PCI I/O modules on 24V |  |  |
| Module for real-time simulator. Parallel digital I/O module with 16x 24V inputs and 16x 24V outputs. They handle up to 0.5A on output ports. Blockset drivers for MATLAB R2018b, drivers, cabling and connector board. | YES | the participiant indicates YES/NO |
| 2 pcs of Extensions for TTL PCI I/O module for PWM/CAP/Interrupt modes |  |  |
| Module for real-time simulator. The TTL module will have at least 4x PWM TTL, 4x Capture TTL and 2x Interrupt TTL I/O. Based on FPGA. Blockset drivers for MATLAB R2018b, drivers, cabling and connector board. | YES | the participiant indicates YES/NO |
| 1 pc of PCI I/O module with combined DIO, AI and AO |  |  |
| Module for real-time simulator. It will have at least 8x TTL DIO, 32x DIF AI (± 10V, ± 5V) and 16 SE AO voltage. Blockset drivers for MATLAB R2018b, drivers, cabling and connector board. | YES | the participiant indicates YES/NO |
| 1 pc of PCI I/O module for TTL level |  |  |
| Module for real-time simulator. It has at least 64x TTL I/O ports for communication at two voltage levels of 3.3 V and 5.0 V. Based on FPGA. Blockset drivers for MATLAB R2018b, drivers, cabling and connector board. | YES | the participiant indicates YES/NO |
| Software: |  |  |
| Software for I / O modules (Matlab/Simulink driver and program blocks): Software for implementation of I/O ports to demanded modules in Matlab/Simulink (from version R2018a and above). The software is fully compatible with the supplied I/O hardware modules. | YES | the participiant indicates YES/NO |
| DEMO Simulink models for supplied I / O modules: Demo DEMO models for testing and familiarization with supplied I/O modules. | YES | the participiant indicates YES/NO |

The participiant should enter the data in compliance with the technical specification of the offered devices.