Annex no. 1 - Technical specifications of the public contract

Technical specifications

The system for marker-free monitoring of human movements:

The subject of the public contract is the supply of a system for marker-free monitoring of the movements of people in the apartment. In order to monitor the movements of persons and the resulting construction of models depicting the movements of monitored persons, it is not necessary in this system to attach or, in any way, place any marking elements on the monitored persons.

The required system will be installed in a telemetry apartment with a defined floor plan according to Figure 1, which is part of the technical specification, specifically in rooms 214, 215, 216, and 217. Scanning will be performed by cameras located in the apartment. These cameras will provide the measured data to a central computer, which is also part of the delivery. A software application will be installed on the computer, which will evaluate the captured images, both in real-time and with the possibility of evaluating the images after the end of the measurement (offline). Its output will be wire 3D models of people moving in the monitored area.

The system must also be partially portable so that it can be moved to a second telemetry apartment, if necessary. At the same time, it must be recalibrated, without the need for calibration with the participation of the supplier's employee, so that after transfer to another space, its correct function is ensured with compliance with the technical requirements specified below.

The delivery includes the installation of the system on-site, its commissioning, calibration and training of personnel for the use of the system.

Equipment manufacturer:			shall be completed by the participant

Exact type designation of the device:	shall be completed by the participant

The system for marker-free monitoring of human movements must meet at least the following criteria:

| Technical parameters | Minimum required values - must be met! | Equipment offered |  |
| --- | --- | --- | --- |
| A) Technical parameters of cameras: |  |  |  |
| Minimum number of cameras for capturing the entire space | 30 pcs | the applicant shall state YES / NO and complete the value |  |
| Camera type: color camera | YES | the applicant shall state YES / NO |  |
| Picture format | min. 1440 x 1080 pixels | the applicant shall state YES / NO and complete the value |  |
| Resolution | min. 1,6 Mpix | the applicant shall state YES / NO and complete the value |  |
| Maximum pixel size | 3,45 x 3,45 µm | the applicant shall state YES / NO and complete the value |  |
| Pixel Depth: min. 8, 12 , 16 and 24 bit | YES | the applicant shall state YES / NO |  |
| Frame rate: | min. 78 fps | the applicant shall state YES / NO and complete the value |  |
| Communication - output: Ethernet | YES | the applicant shall state YES / NO |  |
| Baud rate | min. 10 Mbit/sec | the applicant shall state YES / NO and complete the value |  |
| Video output type: GigE | YES | the applicant shall state YES / NO |  |
| Synchronization option: external or software | YES | the applicant shall state YES / NO and complete the specific value |  |
| Format of stored image data from cameras: AVI | YES | the applicant shall state YES / NO |  |
| B) Software requirements: |  |  |  |
| Flexion angles detection accuracy | at least 5 degrees | the applicant shall state YES / NO and complete the value |  |
| Frame Rate analysis outputs | at least 50 Hz | the applicant shall state YES / NO and complete the value |  |
| Delay between video and analysis output | max. 40 ms | the applicant shall state YES / NO and complete the value |  |
| Deviation of the position of the person identified by the system from its real position in the apartment: | max. 10 cm | the applicant shall state YES / NO and complete the value |  |
| The system must be able to recognize at least 4 people moving in the monitored area. | YES | the applicant shall state YES / NO |  |
| Video data and analysis output must be synchronized. | YES | the applicant shall state YES / NO |  |
| Export of motion analysis outputs: at least CSV, BVH, FBX | YES | the applicant shall state YES / NO |  |
| The delivery must include a communication protocol and a library of command APIs, enabling programming of own applications outside the supplied software environment. | YES | the applicant shall state YES / NO |  |

The data will be completed by the supplier in accordance with the technical data of the offered equipment.

Figure 1 – Floor plan of 214, 215, 216 a 217