Attachment No. 1 - Technical specification

Technical specification for the “Large scale 3D printer for plastic parts”

Manufacturer of the 3D printer:		to be completed by the participant

Exact type specification of the 3D printer:	to be completed by the participant

Number of pieces: 	1 pc

The 3D printer incl. accessories must meet the following criteria:

| Basic technical parameters | Required values = at least the minimal required values must be met! | Value of the offered device |
| --- | --- | --- |
| The build volume | min. 2 m3 | to be completed by the participant |
| The dimension in any axis | min. 1 m | to be completed by the participant |
| Enclosed build volume with temperature control of air | YES | to be completed by the participant |
| Heated bed with temperature control (at least 100 °C) | YES | to be completed by the participant |
| Number of printing heads | min. 2 | to be completed by the participant |
| Productivity of print at least by one printing head for PLA material | at least 0.5kg per hour | to be completed by the participant |
| Temperature control of printing heads | YES | to be completed by the participant |
| Set of printing nozzles (diameters 0.5mm, 1mm, 1.5 mm) | YES | to be completed by the participant |
| Automatic calibration of bed height | YES | to be completed by the participant |
| Printing layer height | at least 0.2 mm – 1 mm | to be completed by the participant |
| The position step in axes XY | max. 0.05 mm | to be completed by the participant |
| The position step in axis Z | max. 0.01 mm | to be completed by the participant |
| Absolute encoders for position measurement on axis | YES | to be completed by the participant |
| The printing speed | min. 100 mm/s | to be completed by the participant |
| The movement speed in axes XY | min. 200 mm/s | to be completed by the participant |
| Compatible with printing materials PLA, PET, ABS, NYLON. | YES | to be completed by the participant |
| The material bin for uninterrupted print | min. 8 kg of material | to be completed by the participant |
| The touchscreen for control unit | YES | to be completed by the participant |
| The consumables for test of printer, including the large scale printing (at least 20 kg of PLA material) | YES | to be completed by the participant |
| External software for creating of g-code for the printer | YES | to be completed by the participant |
| The maximal outer (installation) dimensions of the machine – 4500 mm x 2500 mm x 3000 mm. It must be possible to install the machine through an opening having these dimensions: Width 2250 mm x Height 2410 mm | YES | to be completed by the participant |

The contractor should enter the data in compliance with the technical specification of the offered device.